STEPS TO JOIN USER TO DOMAIN

- Confirm that the Computer is ready using Clean Up Form and Status of Clean Machines at Hannington Office as of 12/08/2025

- Confirm that the Computer has been formatted

- Check if Windows 11 is installed for devices to gain wireless internet connection to support WAP3

- Confirm that the Computer has the following installed:

- CrowdStrike,

- Chrome,

- Winrar,

- Office 356,

- Adobe Reader.

- Confirm that the Computer was not compromised and is safe

- Request network access to network

- Confirm Computer receives DHCP address from uetcl.com domain

- Confirm the machine has internet access

- Confirm that Windows is activated and updates are running

- Confirm that the Computer matches the barcode

- Block Remote Desktop Protocol (RDP)

- Block Secure Shell (SSH)

- Reset the local administrator name

- Reset the local administrator password

- Confirm User is marked as clean in AD

- Ask SSA to reset User Password in AD

- Join Machine to the domain

- Confirm User is able to login to domain

- Confirm that OneDrive was not compromised and is safe (Ask SSA; investigate one drive files upon login)

- User login and syncing of OneDrive (User Windows Authenticator on the phone)

- Map OneDrive folders to the local PC (Documents, Downloads and Desktop)

- Check Office

- Install Printer Drivers

- Provide the onboarding checklist form to the user

- User signs off to form